# Note

IT4130 – Image Understanding and Processing

1

Lecture 04 – Spatial Filtering

Recapitulation

Point Processing techniques:

Image Negatives
Logarithmic transformation for dynamic range compression
Power – Law transformation
Contrast Stretching
Gray level slicing
Histogram Based processing techniques
		- Histogram Equalization
		- Histogram Modification
Image Differencing
Image Averaging

Image Enhancement

On completion the students will be able to:

Mask Processing Techniques
	-	Linear Smoothing Filter
	-	Median Filter (nonlinear)
	-	Sharpening Filter

Neighborhood

Arithmetic and logical operations may take place on a subset of the image
Typically neighborhood oriented
Formulated in the context of mask operations (also called template, window or filter operations)
Basic concept: let the value of a pixel be a function of its (current) gray level and the gray level of its neighbors (in some sense)

Mask Processing

Sub image is called filter, mask, kernel, template or window
Sub image values are coefficients.
For pixels near the boundary of the image, result may be computed using partial neighborhoods or by padding the input appropriately.

Mask Processing

Convolution and correlation

Correlation
In correlation, the value of an output pixel is computed as a weighted sum of neighboring pixels.

Convolution
A convolution kernel is a correlation kernel that has been rotated 180 degrees.

Correlation of a function with a discrete unit impulse yields a rotated version of the function at the impulse location

Operator

Use of spatial masks for filtering is called spatial filtering
May be linear or nonlinear
Linear filters
Lowpass: attenuate (or eliminate) high frequency components such as characterized by edges and sharp details in an image
Net effect is image blurring
Highpass: attenuate (or eliminate) low frequency components such as slowly varying characteristics
Net effect is a sharpening of edges and other details
Bandpass: attenuate (or eliminate) a given frequency range
Used primarily for image restoration (are of little interest for image enhancement)

Non-linear filters

Nonlinear spatial filters also operate on neighborhoods
Their operation is based directly on pixel values in the neighborhood under consideration
They do not explicitly use coefficient values as in the linear spatial filters
Example nonlinear spatial filters
Median filter: Computes the median gray-level value of the neighborhood. Used for noise reduction.
Max filter: Used to find the brightest points in an image
Min filter: Used to find the dimmest points in an image

Linear filtering

Boundary handling

When the Values of the Kernel Fall Outside the Image

Boundary handling contd.

Zero Padding of Outside Pixels

1984-2012- The MathWorks, Inc

Replicated Boundary Pixels

Lowpass Filter (Averaging Filter)

Lowpass - blur

Using a 5x5 basic low pass filter

Lowpass filters

Also called as averaging filters.
The shape of the impulse response needed to implement a lowpass (smoothing) filter indicates the filter should have all positive coefficients
For a 3x3 mask, the simplest arrangement is to have all the coefficient values equal to one (neighborhood averaging)
scale the result by dividing by 9

Example Filters

Applications

Noise reduction
Random noise results in sharp transitions in gray levels.
Blur edges
Reducing irrelevant details
Smaller than the filter mask
Bridging small gaps in lines and curves

Filter size effects

Explain the importance of
b and c
d
e and f

Averaging masks

Example - Averaging

Highpass - Edge detection

A sample image filtered using a 3x3 basic high pass filter

Non-linear filters

Their operation is based directly on pixel values in the neighborhood under consideration
They do not explicitly use coefficient values as in the linear spatial filters
For example, Order-Statistics filters are spatial filters whose response is based on ordering (ranking) the pixels contained in the image area encompassed by the filter

Order-Statistics Filters

Median filter
Computes the median gray-level value of the neighborhood. Used for noise reduction.
Max filter
Used to find the brightest points in an image
R = max{zk | k =1,2,...,9} 
Min filter
 Used to find the dimmest points in an image
R = min{zk | k =1,2,...,9}
Midpoint filter
Alpha-trimmed mean filter

Median filter

Process is replaces the value of a pixel by the median of the gray levels in region Axy of that pixel:

Smoothing with median

Smoothing
Averaging
One problem with the lowpass  filter is it blurs edges and other  sharp details.
Median
 If the intent is to achieve noise reduction, one approach can be to use median filtering
The value of each pixel is replaced by the median pixel value in the neighborhood (as opposed to the average)
Particularly effective when the noise consists of strong, spike like components (impulse noise/ salt and pepper noise) and edge sharpness is to be preserved
The median m of a set of values is such that half of the values are greater than m and half are less than m
To implement, sort the pixel values in the neighborhood, choose the median and assign this value to the pixel of interest
Forces pixels with distinct intensities to be more like their neighbors

Example - Median filtering

Max and Min filter

Using the 100th percentile results in the so-called max filter, given by





The 0th percentile filter is min filter:

This filter is useful for finding the brightest points in an image. Since pepper noise has very low values, it is reduced by this filter as a result of the max selection processing the subimage area Axy.

This filter is useful for finding the darkest points in an image. Also, it reduces salt noise as a result of the min operation.

Example – Min filtering

Midpoint filter

The midpoint filter simply computes the midpoint between the maximum and minimum values in the area encompassed by the filter:

Note: This filter works best for randomly distributed noise, like Gaussian or uniform noise.

Alpha-trimmed mean filter

Suppose that we delete the d/2 lowest and the d/2 highest gray-level values of r(i,j) in the area Axy.
Let rk(i,j) represent the remaining mn-d pixels. And averaging these remain pixels is denoted as:

Where the value of d can range from 0 to mn-1. When d=0, It is arithmetic mean filter and d=(mn-1)/2 is a median filter. It is useful for the multiple types of noise such as the combination of salt-and-pepper and Gaussian noise.

Noise

Any real world sensor is affected by a certain degree of noise, whether it is thermal, electrical or otherwise. 
S = r + noise

Noise (Contd.)

Gaussian Noise




Salt and Pepper Noise

Mean Filters

These are simple methods to reduce noise in spatial domain.
Arithmetic mean filter
Geometric mean filter
Harmonic mean filter
Contraharmonic mean filter
Let Axy represent the set of coordinates in a rectangular subimage window of size mxn, centered at point (x,y).

ref(1) pages 231-233

Arithmetic mean filter

Compute the average value of the corrupted image r(i,j) in the area defined by Ax,y.
The value of the restored image       at any point (x,y)

Note: Using a convolution mask in which all coefficients have value 1/mn. Noise is reduced as a result of blurring.

Geometric mean filter

Using a geometric mean filter is given by the expression

Harmonic mean filter

The harmonic mean filter operation is given by the expression

Contraharmonic mean filter

The contraharmonic mean filter operation is given by the expression

Where Q is called the order of the filter. This filter is well suited for reducing or virtually eliminating the effects of salt-and-pepper noise.